# VLANs

VLAN 10 IT PC1 - PC7 (f0/1 - f0/08 interfaces on S1) PC8- PC9 (f0/9 -10 interface on S2)

VLAN 20 Finance PC18 - PC 19 (f0/9 - f0/10 interface on S1) PC10 - PC17 (f0/1 -f0/8 interface on S2)

VLAN 30 Marketing PC20-PC22, PC26-PC29 (f0/1-f0/7 on S3) PC24-PC25 (f0/1 -f0/2, f0/10 on S4)

VLAN 40 R&D PC 30, PC35- PC36 (f0/8-f0/10 on S3) PC30-PC33, PC37-PC38 (f0/3-f0/7 on S4) PC34, PC39 (f0/1 -f0/2 on S5)

VLAN 50 Sales PC40, PC45 (f08 -f0/9 on S4) PC41-PC44 and PC46-PC49 (f0/3 -f0/10 on S5)

VLAN 99 Unused (All interfaces that are not in use)

1. give the switch a hostname

2. make VLANs needed for the switch

3. go into the switch interface and put the right interface in the right VLANs

4. move any unused interfaces into VLAN 99

5. put g0/1 and g0/2 on a trunk interface if in use

The g0/1 and g0/2 interfaces will act as trunk interfaces If the interfaces are in use.

Note: The two gigabit ethernet interfaces that are in VLAN1 we work fully as trunks after basic router configuration is completed which will include inner VLAN routing.